Router Firmware Upgrade Guide

Note

- To prevent the router from being damaged:

- Ensure that the firmware is applicable to the router. Generally, the firmware upgrade file suffix with .bin.

- When you are upgrading the firmware, do not power off the router.

- It is recommended to connect the router`s LAN port to your computer`s Ethernet port directly via an Ethernet cable when performing a firmware upgrade. Wireless connection for upgrading may cause the upgrade to fail.

Method

- Ensure the router's power cable is securely connected. Connect your computer to the router via an Ethernet cable: plug one end into the router's LAN port and the other end into your computer's Ethernet port.

- Log in to the router's web management page in your browser (address: tendawifi.com).

- If you have previously set a router login password, the login window will pop up automatically. Please enter your password to log in. If you forget your password, click the Forgot password? next to the login window and follow the on-screen instructions to verify or reset your password.

- Navigate to System Settings > Firmware Upgrade

- Click Local Upgrade in the line of the router to be upgraded. The following figure if for reference only.

- Click Select a File. Find the firmware file downloaded previously (suffixed with .bin).

- Click Upgrade.

- Wait until the upgrade completes. Access the Firmware Upgrade page again and check whether the upgrade is successful based on current firmware Version.

- --END

If you encounter any issues or have questions during the update, please send your product model, ISP update file version and detailed fault description to support.fwa@tenda.cn for technical support.